Mobile Data Terminals (MDTs) in Indian Emergency Services

1. Core Functionality & UI

MDTs are ruggedized tablets or in-vehicle computers used by police, fire, and ambulance units ( ERSS 112, UP100).

Tri-Panel Layout:

  - Case List: Shows active incidents with ID, type, and timestamp.

  - Interactive Map: Displays the incident location, real-time vehicle position (GPS), and "Get Path" navigation.

  - Case Details: Lists caller info, priority (color-coded), and subtype (e.g., "Robbery").

Action Keys: Large buttons for Accept/Reject/Complete, camera uploads, and VoIP calls.

2. Key Technology

- CAD Integration: Incidents logged at the Public Safety Answering Point (PSAP) are automatically pushed to the nearest MDT based on GPS proximity.

- Navigation & GIS: Uses MapmyIndia or Google Maps for turn-by-turn routing, including specific "house number" searches and POIs like hydrants or hospitals.

- Bidirectional Communication: Supports text, image, and video chat between field crews and the control room.

- Connectivity: Operates via GPRS/4G with an SMS fallback to ensure reliability in low-network areas.

3. Major Systems & Vendors in India

| System/Agency | Technology Provider | Scope/Usage |
| --- | --- | --- |
| ERSS (112 India) | C-DAC | Nationwide indigenous platform for police, fire, and rescue. |
| Dial-100 (MP/UP) | Hexagon & Mahindra | Integrated CAD/MDT system; handles thousands of patrol vehicles. |
| 108 Ambulance | InfoTrack / GVK EMRI | Specialized for medical dispatch, patient tracking, and hospital routing. |

- Figure1: Example of an Indian emergency MDT UI. Active incidents are listed on the left; a map with the incident marker in the center; detailed case data (ID, type, caller, etc.) and action buttons (Accept/Reject, text, camera, route) on the right.

Figure2: Another MDT screen (the “FIRE” app theme). It shows one active case with its map and details. Buttons allow a voice call to the caller, SMS chat, camera for photos, and a “Report” button (likely for incident updates.

India’s PSAP Dashboard Systems

1. System Overview

India is transitioning to a unified 112/ERSS (Emergency Response Support System). This system consolidates various agencies (Police-100, Fire-101, Ambulance-108) into a single technological framework.

- Primary Platform: C-DAC’s ERSS software (indigenous "Total Solution").

- Architecture: A centralized State PSAP (Public Safety Answering Point) linked to District Coordination Centers (DCCs) and field units.

2. Typical Dashboard UI Layout

The dispatcher’s "Single Pane of Glass" is usually a multi-panel interface spanning several monitors:

- Center Panel (GIS Map): The heart of the UI. It displays real-time GPS locations of all emergency vehicles and geocoded incident pins.

- Left Panel (Incident/Caller Info): Pops up automatically when a call lands. Shows Caller ID, phone number, and auto-detected location (via cell tower/GPS).

- Right Panel (Resource Status): A roster of all units (Police, Fire, EMS). Units are color-coded (e.g., Green for Available, Blue for En Route, Red for On Scene).

- Bottom Panel (Event Log): A scrolling list of active cases with status updates (Pending, Dispatched, Closed).

3. Key Functional Modules

| Module | Functionality |
| --- | --- |
| CAD (Computer-Aided Dispatch) | Logs calls, creates "events," and provides automated unit recommendations based on proximity. |
| GIS (Mapping) | Integrates road networks and Points of Interest (POIs) like hospitals, police stations, and fire hydrants. |
| Communication Gateway | Handles multi-channel distress signals: Voice, SMS, Email, Panic SOS, and Web requests. |
| MDT Integration | Pushes incident data directly to the Mobile Data Terminal (Tablet) in the rescue vehicle. |

4. Operational Workflow

- Intake: A call taker receives the signal; the system geolocates the caller on the map.

- Verification: The incident is classified (e.g. "Medical Emergency") and an SOP is generated.

- Dispatch: The dispatcher identifies the nearest available unit on the map and "pushes" the case to their MDT.

- Monitoring: The dispatcher tracks the unit’s travel in real-time. Communication (chat/VoIP) is recorded in the case log.

- Resolution: Once the field unit marks the task as "Complete," the dashboard updates, and the unit returns to "Available" status.

MOM-04/02/26

Simulation evaluating the system

Any audio data to check the performace of caller agent